;**********************************************************************

;                                                                     *

;    Filename:	    Pickey629Egg.asm                                 *

;    Date: 2008 May 6                                                 *

;    File Version: 0                                                  *

;                                                                     *

;    Author:   JR7DUT                                                 *

;    Company:  DUT Brewery                                            *

;                                                                     *

;                                                                     *

;**********************************************************************

;                                                                     *

;    Files required:                                                  *

;                                                                     *

;                                                                     *

;                                                                     *

;**********************************************************************

;                                                                     *

;    Notes:                                                           *

;         changed to TMR0 Polling from INT(002>004)                               *

;                 create file for release

;		  File is same as Pickey629Egg004                                           *

;                                                                     *

;                                                                     *

;**********************************************************************

list      p=12f629        		   ; list directive to define processor

#include <p12f629.inc>        ; processor specific variable definitions

errorlevel  -302              ; suppress message 302 from list file

__CONFIG   _CP_OFF & _CPD_OFF & _BODEN_OFF & _MCLRE_OFF & _WDT_OFF & _PWRTE_ON & _EXTRC_OSC_NOCLKOUT

;***** VARIABLE DEFINITIONS

MKn			EQU	0x31		; MARK count

NEXT		EQU	0x32		; index for next Dash or Dot

fMK			EQU	0x33		; MARK Flag

;**********************************************************************

ORG     0x000		; processor reset vector

nop							; nop required for icd

goto    main					; go to beginning of program

;===========================================================================

; Initialize Parameters

main	bcf			STATUS,RP0		; Bank 0

clrf			GPIO			; Clear GPIO

movlw		007h			; Set GPIO<2:0> to Digital IO

movwf		CMCON		;

clrf			MKn			; Initialize MKn

clrf			fMK				; Initialize fMK

clrf			NEXT			; Initialize NEXT

; Bank1 Parameters

bsf			STATUS,RP0		; Bank 1

bcf			TRISIO,2			; Output Pin assignment(Temporary fortest)

bcf			TRISIO,4			; Output Pin assignment

movlw		b'00000111'		; Set OPTION_REG

movwf		OPTION_REG			; bit7=0		weak pull-up Enable

; bit6		no concern (GP2/INT edge select)

; bit5=0		TMR0 clock source to

;			Internal instruction cycleclock(CLKOUT)

; bit4		no concern (EXT clock edge select)

; bit3=0		Prescalor  to TMR0

; bit<2,0>=<1,1,1>	prescalor rate 1:256

bcf			STATUS,RP0		; Bank 0

;============================================================================

; Beginning of Main routine

; Polling DOT or DASH comes in.

lp_ini		btfss		GPIO,0				; DOT key off ?

goto		iniDOT				; No, GPIO,0=L. DOT request.

btfss		GPIO,1				; DSAH key off ?

goto		iniDASH				; No, GPIO,1=L. DASH request.

goto		lp_ini				; when TMR1 Interrupt

; DOT key on

iniDOT			bsf			GPIO,4			; Start Mark

movlw		d'219'			; TMR0 set value

bcf			INTCON,T0IF		; Clear over flow flag

movwf		TMR0			; Start new count up

movlw		d'1'

movwf		MKn				; MKn=1

movlw		b'00000001'

movwf		fMK				; fMK=<0,1>

goto		lp_Ton

; DASH key on

iniDASH			bsf			GPIO,4			; Start Mark

movlw		d'219'			; TMR0 set value

bcf			INTCON,T0IF		; Clear over flow flag

movwf		TMR0			; Start new count up

movlw		d'3'

movwf		MKn				; MKn=3

movlw		b'00000010'

movwf		fMK				; fMK=<1,0>

goto		lp_Ton

; After initial output wait following input.

lp_Ton		btfss		GPIO,0				; DOT key off ?

call			NX_DOT				; No, GPIO,0=L. DOT key on.

btfsc		INTCON,T0IF			; Watch TMR0 overflow Flag

goto		T0up				; T0IF if set. Go to time up treatment.

btfss		GPIO,1				; DSAH key off ?

call			NX_DASH			; No, GPIO,1=L. DASH key on.

btfsc		INTCON,T0IF			; Watch TMR0 overflow Flag

goto		T0up				; T0IF if set. Go to time up treatment.

goto		lp_Ton				;

; DOT key on sequence;;;;;;subroutine;;;;;;;;

NX_DOT			btfsc		fMK,0				; not in DOT output sequence ?

goto		Exit_NX_DOT			; fMK,0=1, btfsc comes No. Nothing done.						movf		NEXT,F				; fMK,0=0, btfsc comes Yes. No DOT output.

btfss		STATUS,Z			; NEXT empty ?

goto		Exit_NX_DOT			; No, NEXT is set already.

movlw	b'00000001'			; Yes, set NEXT

movwf	NEXT				;

Exit_NX_DOT	return

; DASH key on sequence;;;;;;subroutine;;;;;;

NX_DASH		btfsc		fMK,1				; not in DASH output sequence ?

goto		Exit_NX_DASH			; fMK,1=1, btfsc comes No, Nothing done.

movf		NEXT,F				; fMK,1=0, btfsc comes Yes. No DASH output.

btfss		STATUS,Z			; NEXT empty ?

goto		Exit_NX_DASH			; No, NEXT is set already.

movlw	b'00000010'			; Yes, set NEXT

movwf	NEXT				;

Exit_NX_DASH	return

;**************************************************************************************************************

; TMR0 time up Sequence

; TMR0 Re-countup

T0up		movlw		d'219'			; TMR0 set value

bcf			INTCON,T0IF		; Clear over flow flag

movwf		TMR0			; Start new count up

; check if MARK on ?

movf		MKn,F

btfsc		STATUS,Z		; MKn=0,Z=1,btfsc=No,next row.  MKn=1,2,3,Z=0,Jump.

goto		ckNX0			; Space after MARK finished, GO for NEXT treatment.

; Case of MARK on

decfsz		MKn,F		; check following is MARK or SPACE

; following is MARK continue

goto		Exit_inSQ		; following is MARK, only TMR0 restart

; following is SPACE

bcf			GPIO,4			; Clear output

goto		Exit_inSQ

; NEXT treatment

; NEXT=DOT ?

ckNX0		btfss			NEXT,0			;

goto			ckNX1			; NEXT is not DOT, check DASH

bsf			GPIO,4			; Yes, NEXT is DOT. Start Mark.

movlw		d'1'

movwf		MKn				; MKn=1

movlw		b'00000001'

movwf		fMK				; fMK=<0,1>

clrf			NEXT

goto			Exit_inSQ

; NEXT DASH ?

ckNX1		btfss			NEXT,1

goto			ckGP0			; NEXT is niether DOT nor DASH.

bsf			GPIO,4			; Yes, NEXT is DASH. Start Mark.

movlw		d'3'

movwf		MKn				; MKn=3

movlw		b'00000010'

movwf		fMK				; fMK=<1,0>

clrf			NEXT

goto			Exit_inSQ

; NEXT is neither DOT nor DASH, Check on key.

; If key on, DOT after DOT, DASH after DASH.

; 	because if opposite MARK has already entrained, NEXT must has existed.

ckGP0		btfsc			GPIO,0			;

goto			ckGP1			; GPIO,0=H, Check DASH key.

bsf			GPIO,4			; Yes, NEXT is DOT.

movlw		d'1'

movwf		MKn				; MKn=1

movlw		b'00000001'

movwf		fMK				; fMK=<0,1>

goto			Exit_inSQ

ckGP1		btfsc			GPIO,0			;

goto			endSQ			; no NEXT & no on key, Sequence finished.

bsf			GPIO,4			; Yes, NEXT is DASH.

movlw		d'3'

movwf		MKn				; MKn=3

movlw		b'00000010'

movwf		fMK				; fMK=<0,1>

goto			Exit_inSQ

Exit_inSQ		goto		lp_Ton			;

endSQ	clrf			MKn			;

clrf			fMK				;

clrf			NEXT			;

goto			lp_ini			; end of sequence. clear parameters and return to ini loop

END                       ; directive 'end of program'